[image: image1.jpg]TREEHAVEN

technologies





24 Village Pointe Drive


Powell, OH 43065



Phone: 614.791.8843



Fax: 614.789.0252



www.treehavenvision.com 



A&E Specification:

CAMERA CONTROLLER



SYSTEM DESCRIPTION

The High Speed PTZ Camera Controller must feature a proportional 3-Axis joystick capable of generating 8 bit resolution for each axis pan, tilt and zoom, manual focus and iris control, a minimum of four programmable user soft keys, camera pre-position programming and control, surveillance tour programming, and graphic VF display. The camera controller must be suitable for both desktop and portable handheld use and include a laminated display and keypad overlay, weather and chemical resistant Elastomer keypad with sealed legends, and a sealed joystick control. The camera controller must provide a connector for accepting a standard PS2 keyboard for system programming. The camera controller shall operate from a 5 VDC power supply and be fully compatible with the RVSHD6 series High Speed PTZ Dome.

MANUFACTURED UNITS

RVS3000 - Industrial Desktop/Handheld 3-Axis Proportional PTZ Camera Controller. Includes manual focus and iris controls, 4 user-programmable soft keys, camera preposition programming and control (PreSets), surveillance tour programming and control (VectorScan) and graphic VFD display. 5-volt operation. 

SYSTEM PERFORMANCE

The High Speed PTZ Camera Controller shall include, as a minimum, the following features/functions/specifications:

The High Speed PTZ Camera Controller and its components shall be thoroughly tested before shipping from the manufacturer’s facility.

The High Speed PTZ Camera Controller shall be designed for both desktop and portable hand-help use. The controller shall be of a size and weight to be held with one hand.

The High Speed PTZ Camera Controller shall utilize flash memory storage of programmed settings. Removing power from the unit shall not cause any programmed settings to be lost.

The High Speed PTZ Camera Controller shall include a VFD graphics display, a three axis proportional joystick control with 8 bit resolution per axis, and an elastomer keypad with a solid backing for moisture resistance.  The elastomer keypad shall contain laser etched key designations and a protective coating. The controller shall include a scratch and smudge resistant laminated overlay to protect the VFD display and provide a weather resistant seal.

The High Speed PTZ Camera Controller shall include a 5-pin mini-DIN connector to accept a standard PS/2 keyboard for system programming.  

The High Speed PTZ Camera Controller shall be easy to use and include the following keys: CAMERA and MONITOR keys for camera call-up and control, PRESET key for initiating preprogrammed Presets, a FUNCTION key for accessing the softkey menus, a CLEAR key to clear entries, an ENTER key complete user entries, and four softkeys to access the softkey functions. 

The High Speed PTZ Camera Controller shall feature the following softkey menu selections:

1. Iris

a. Open

b. Close


c. Auto

2. Focus


a. Near


b. Far

3. MultiCall


a. Run


b. Stop All


c. Start


d. Stop


e. Hold


f. Step Forward


g. Step Backward


h. Continue

4. VectorScan


a. Run VectorScan Once


b. Run VectorScan Continuous

5. Tour


a. Run Tour 1


b. Run Tour 2


c. Run Tour 3

6. List


a. Preset


b. VectorScan


c. VectorScan Continue


d. Scroll Up


e. Scroll Down


f. Display Sector


g. Page 


h. Exit

7. Home

8. Freeze


a. Freeze


b. Unfreeze

9. Program


a. PreSets


b. Tour



i. Start Tour



ii. End Tour



iii Run Tour


c. Function Key

10. Information


a. Camera ID



i. ID ON



ii. ID OFF


b. Azimuth



i. Azimuth ON



ii. Azimuth OFF


c. Error Log



i. Error Log ON



i. Error Log OFF

11. Diagnostics


a. Offset



i. Offset ON



ii. Offset OFF


b. Actual



i. Actual ON



ii. Actual OFF


c. Auto Zero


d. Software Revision 

12. Setup


a. JPD101


b. JPD200


c. Address


d. Protocol

MECHANICAL SPECIFICATIONS

The High Speed PTZ Camera Controller must have the following mechanical specifications:

Unit Dimensions (H X W X D):
5.5" x 8.7” X 5.6"

Unit Weight:



1.5 lbs. 

ELECTRICAL POWER REQUIREMENTS

The High Speed PTZ Camera Controller must have the following electrical specifications:

Input Voltage:
5 VDC

Power Consumption:
15 Watts

ENVIRONMENTAL CONDITIONS

The High Speed PTZ Camera Controller shall be designed to meet the following environmental conditions:

Operating Temperature:

32° to 122° F (0° to 50° C)

Humidity:


0-95%, Non-Condensing
Rev. 1.0 
1
19-May-11


